# MINIMUM SPECIFICATIONS FOR BOSSAIR BA440 RECIPROCATING AIR COMPRESSOR

Quantity of one (1) Boss Model BOSSAIR BA440 to include the following as standard equipment:

| COMPRESSOR |  |
| --- | --- |
| - BOSS single stage, air cooled, 4-cylinder, pressure lubricated reciprocating air compressor. Aluminum compressor assembly for lighter weight and higher thermal efficiency - No cast iron compressor assembly acceptable CFM/PSI; RPM; GPM; HYD PSI. 30/100 (150 max); 1,150; 8; 1,850 (2,100 max). 35/100 (150 max); 1,400; 10; 1,850 (2,100 max). 40/100 (150 max); 1,650; 12; 1,850 (2,100 max). - Integrated hydraulic system cooler to handle air compressor GPM flow - Compressor to be pressure lubricated via oil pump - Splash lubrication only is not acceptable - Integrated solenoid and relief valve incorporated into the compressor package Type: Above deck, hydraulically driven reciprocating air compressor rated at 40 CFM up to 100 PSI; BA440 system is to run intermittently |
| Capacity: Up to 85 CFM @ 110 PSIG continuously |  |
| Drive System: Direct coupled, gear-type hydraulic motor to compressor input shaft. No belt drive allowed |  |
| Air Feed: 200 psi 1/2” – 3/4” air feed downstream of the BA 440 air out is required |  |
| Air Reservoir: Minimum 22 Gallon Air Reservoir required |  |
| Operating Slope: 20 deg. Maximum operating slope in any direction |  |
| Manual: To include operation, maintenance and warranty information |  |
| FINISH |  |
| Enclosure to be durable, light weight, powder-coat painted and rust resistant aluminum. Galvanneal sheet metal not acceptable |  |
| WARRANTY |  |
| - 18 months – see warranty statement |  |
| COOLING |  |
| - 12VDC electric 14” fan assembly integrated into package for compressor head cooling and single pass hydraulic system cooler - Cooling air intake must not see air temperatures above ambient - Cooling air discharge must have 10” clearance from any obstructions - Ambient temperature for operation should be below 100°F (38°C) - The machine may experience high temperature shutdown above this level |  |
| SERVICE |  |
| Routine maintenance items accessible without removal of canopy |  |
| PACKAGING |  |
| Dimensions | 26 ½” L x 19 ⅛”H x 19 ¾”W |
| Weight | Not to exceed 180 lbs |